CLI Prompts -

| CMD | brief |
| --- | --- |
| request license info | to show the device license details |
| request support check | Check the Support Information |
| request support info | Check the Support information |
| show device-certificate status | Shows the management certificate **** |
| request license fetch | this will fetch all the license from server |
| show user server-monitor state all |  |
| show user group-mapping statistics | show the user group mapping |
| show user group list | show the user groups if any |
| show user ip-user-mapping all | Show the users list mapped with IP |
| show global-protect-gateway gateway | Show list of GlobalProtect gateway configuration |
| show global-protect-gateway flow | tunnel configs for global protect |
| show system info | shows the uptime, serial number |
| show system environmentals | e.g. power supply failures |
| show ntp | shows the ntp server details |
| request restart system | reboots the whole device |
| show session info | packet rate, number of sessions, fastpath active, etc |
| show session id <id> |  |
| show interface { all ¦ <interface-name> } |  |
| show routing route | routing table (all routes) |
| show routing fib | forwarding table (only used routes) |
| show routing protocol <protocol> |  |
| show neighbor interface { all ¦ <interface-name> } | IPv6 neighbor cache |
| show arp { all ¦ <interface-name> } |  |
| show mac all | only with layer 2 interfaces |
| show jobs all |  |
| show jobs id <id> |  |
| show running resource-monitor | resource statistics |
| show system resource follow | ="top", CPU usage and processes |
| show system disk-space | ="df -h" |
| debug software restart <service> | Restart a certain process |
| show system statistics session show system statistics <application> |  |
| find command keyword <word-to-search-for> |  |
| ping host 8.8.8.8 |  |
| ping inet6 yes source 2003:51:6012:120::1 host 2a00:1450:4008:800::1017 | ipv6 ping |
| traceroute host 8.8.8.8 |  |
| ping host ip.google.com | resolve dns |
| show vpn gateway | check VPN issues |
| show vpn ike-sa | check vpn issues |
| show vpn flow name <value> |  |
| show high-availability state | to show the high availability |
| show high-availability all | to view the sync status |
| request high-availability state suspend | this will suspend the active device to passive |
| request high-availability state functional | force the active device to transition to passive |